Monocrystalline Structure

Synthetic Diamond Powders

# Background

Our monocrystalline synthetic diamond powders come directly from manufacturing plant.  Our technology allows the production of different grades of diamonds to suite most applications.

Very strict process control insures consistent premium quality of synthetic diamond powders.  The resin and metal bond system diamonds are available in variety of grades and sizes, along with the mesh sized diamonds of different hardness, all in compliance with the US Bureau of Standards. The metal bond system diamonds are also available with copper or nickel coating.

These diamonds are the best suited for abrasive wet and dry superfinishing and lapping of nonferrous metals, nonmetallics and composite materials, such as Tungsten Carbide, Titanium alloys, Glass, Ceramics and Cermets.  They are also well suited for honing and polishing die steels and cast irons, metal plating of diamond tools, dressing and truing silicon carbide grinding wheels.

# Product Availability and Application Reference Chart

Our stock inventory, with the reference to the process, is listed below.  Other grades and sizes are also available.

| Process | Grade No. | US Standard Micron Range | Mesh Sizes |
| --- | --- | --- | --- |
| Polishing | 1/4 | 0-1/2 | 60,000 |
|  | 1/2 | 0-1 | 25,000 |
|  | 1 | 0-2 | 10,000 |
|  | 2 | 1-3 | 9,000 |
|  | 3 | 2-4 | 8,500 |
|  | 4 | 2-6 | 6,000 |
|  | 6 | 4-8 | 3,000 |
| Lapping | 9 | 6-12 | 1,800 |
|  | 12 | 8-15 | 1,500 |
|  | 15 | 10-20 | 1,200 |
|  | 17 | 15-20 | 1,100 |
|  | 20 | 15-25 | 1,000 |
|  | 25 | 20-30 | 880 |
|  | 30 | 22-36 | 600 |
|  | 35 | 30-40 | 500 |
|  | 45 | 36-54 | 450 |
| Grinding | 400 | 35/45 | 325/400 |
|  | 320 | 45/53 | 270/325 |
|  | 240 | 53/63 | 230/270 |
|  | 220 | 63/75 | 200/230 |
|  | 180 | 75/90 | 170/200 |
|  | 150 | 90/106 | 140/170 |
|  | 120 | 106/125 | 120/140 |
|  | 110 | 125/150 | 100/120 |
|  | 105 | 150/180 | 80/100 |
|  | 100 | 180/250 | 60/80 |
|  | 60 | 250/300 | 50/60 |
|  | 46 | 300/425 | 40/50 |

Note: Nickel and Copper coatings are available per request.

Bulletin WIUKA\DM1 Rev. 4   6.23.2002